# LNG Gas Composition & Impurity Specification

| Parameter | Unit | Specification Limit | Test Method Reference | Remarks |
| --- | --- | --- | --- | --- |
| Methane (CH₄) | mol% | ≥ 85.0 | ASTM D1945 (GC) | Primary component |
| Ethane (C₂H₆) | mol% | ≤ 10.0 | ASTM D1945 | Controlled for heating value |
| Propane+ (C₃⁺) | mol% | ≤ 5.0 | ASTM D1945 | Heavy hydrocarbon limit |
| Nitrogen (N₂) | mol% | ≤ 1.0 | ASTM D1945 | Inert content limit |
| Carbon Dioxide (CO₂) | ppmv | ≤ 50 ppmv | ASTM D1945 | Prevent freezing |
| Hydrogen Sulfide (H₂S) | ppmv | ≤ 4 ppmv | ASTM D5504 | Corrosion & safety |
| Total Sulfur | ppmv | ≤ 30 ppmv | ASTM D6667 | Environmental compliance |
| Oxygen (O₂) | ppmv | ≤ 10 ppmv | ASTM D1945 | Safety limit |